NILA-AV / DHRUV

HOW, WHAT & WHY

NETWORK SETUP

GUIDE

A practical Dhruv network guide explaining what is needed, why it is

needed, how to configure it, and how to prove it works.

For customer IT teams, network engineers, installers and Dhruv administrators.

Use the baseline profile, then add only the streaming and management profiles selected for the site.

The practical starting point

For the repository-documented HTTPS player design, plan device-initiated TCP 443 to the approved Dhruv service

and media destinations, plus the site’s DNS, time and address services. Do not create internet port-forwards to

screens for ordinary signage. Streaming requirements depend on the transport and endpoint actually selected.

[R2–R5, S1]

Document control                   Record

Reference                          DHRUV-NET-SETUP-001

Version / prepared                 v1.1 - How / What / Why edition

Scope                              SoC signage, approved players, Amino H200, live streaming, hosted or local services

Evidence basis                     Repository source and current primary vendor/protocol documentation

Complete the site register and pass the relevant commissioning tests before

Release condition

acceptance

Important: This is a setup and approval guide, not a certificate that a live server, stream, screen model or deployment

has passed testing. No firewall, server or player configuration was changed while preparing it.

Source snapshot: satinder-source/dhruv-platform

9a372468f73656754fdff5f8b83fddd5e6ad8907

Version 1.0 | 16 September 2026   •   1 / 28

NILA-AV / DHRUV                NETWORK SETUP & LIVE STREAMING

01 / NETWORK SETUP

HOW / WHAT / WHY

How to use this guide

Every network requirement in this guide should be read in four steps: WHAT Dhruv needs; WHY the service is

needed; HOW the customer IT team or installer should configure it; and PROVE IT - the evidence that confirms the

path works. This prevents a port list from being treated as a blanket firewall instruction.

WHAT                        WHY                            HOW                            PROVE IT

DNS resolution              Dhruv uses hostnames for       Provide a reachable site       Resolve the approved

CMS, Player and approved       DNS resolver. Permit the       Dhruv hostname from the

media services; hard-          Player VLAN to query only      Player network and

coded IP addresses are         the approved resolver(s).      record the returned

not a durable design.                                         addresses and timestamp.

Accurate time               TLS validation, schedules,     Provide DHCP-supplied or       Compare device time with

authentication leases and      explicitly configured          the approved source;

operational logs depend        NTP/time service. Keep         verify timezone and

on correct time.               UDP 123 limited to the         scheduled playback

approved time source           behaviour.

where applicable.

Outbound HTTPS - TCP        The repository-                Permit device-initiated        From the Player VLAN,

443                         documented Player              TCP 443 only to approved       resolve the hostname,

design uses HTTPS for          Dhruv and media                complete TLS, enrol the

enrolment/content/contr        destinations. No WAN-to-       device, retrieve an

ol contracts and secure        screen port-forward is         assignment and observe a

content delivery.              required for normal            successful heartbeat.

signage.

Live-stream transport -     A live encoder or Player       Open only the exact            Run the real stream on

only when used              may use a transport other      protocol, direction,           the target device and

than HTTPS, such as SRT,       host/group and                 record sustained

RTSP/RTP, RTMPS or             configured port(s)             playback, bitrate/packet-

multicast.                     selected for that stream.      loss evidence and

reconnect behaviour.

Local display control -     Some professional              Keep control traffic on a      Use Test

only when supported         displays expose                management VLAN and            Connection/status first,

manufacturer control           allow only the Dhruv           then a harmless

over the LAN. This is          controller to the exact        supported command;

separate from ordinary         display IP/port supported      record the real device

cloud Player traffic.          by that adapter.               response and audit event.

Fast decision: what does this site actually need?

Standard signage normally starts with IP addressing, DNS, time and outbound HTTPS. Web URL content adds access

to specifically approved external origins. Live streaming adds only the selected stream transport. Local hardware

control adds only the manufacturer-specific LAN rule. Remote support is a separate optional service. Do not

combine these profiles into an “open everything” firewall rule.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                   NETWORK SETUP & LIVE STREAMING

Start here: choose the right profile

There is no single “open all Dhruv ports” rule. Approve the core profile first, then the smallest set of optional

profiles needed by the installation.

Profile                   Select when                                    Network addition

Players download assigned content and          Approved HTTPS destinations on TCP 443; site DNS, NTP

A • Standard signage

report to Dhruv.                               and DHCP as applicable.

The selected device/app accepts an             TCP 443 to manifests, media, redirects and required

B • HTTPS live playback

approved HLS or DASH service.                  key/licence services.

C • Contribution / low    An encoder, gateway or decoder uses RTMP,      Only the configured receiver/control/media ports; see

latency                   RTSP or SRT.                                   sections 09–10.

One headend distributes channels to            Assigned multicast groups and UDP ports, with IGMP-

D • LAN IPTV multicast

multiple local receivers.                      aware switching; section 11.

The selected application actually implements

E • WebRTC                                                               Its ICE/STUN/TURN and media configuration; section 12.

WebRTC.

H200 provisioning uses Amino Orchestrate /

F • Amino management                                                     The separate vendor destination matrix in section 13.

Resolve.

A separately approved support service is       Its named servers and narrowly scoped management

G • Remote support

installed.                                     rules; section 14.

One-page IT handover: minimum Dhruv network request

Give this summary to the customer IT/network team before installation. Values marked “site-specific” must be

completed from the approved deployment; they are not universal Dhruv defaults.

WHAT                            WHY                               HOW                             PROVE IT

Player network                  Provide a managed wired           Stable addressing,              Connect one Player on

VLAN/subnet for fixed             isolation and                   the final VLAN and verify

Players/screens where             troubleshooting are             address, gateway, DNS

practical. DHCP                   easier than on guest            and time.

reservations may be used          networks.

where operationally

helpful.

Outbound HTTPS                  TCP 443 from Players to           Enrolment, content,             Enrol one device and

approved Dhruv/media              schedules,                      complete a content

FQDNs, including the              heartbeat/control               publish/heartbeat cycle.

repository-documented             contracts and HTTPS

production Player origin          media require secure

https://player.dhruv.org.u        outbound connectivity.

k where that is the

deployed target.

CMS access                      HTTPS TCP 443 from                Operators need secure           Sign in from an authorised

authorised administrator          browser access to               admin network and

networks to the deployed          manage content, screens         complete a read-only

CMS origin, repository            and schedules.                  screen/status check.

evidence referencing

https://cms.dhruv.org.uk.

DNS / time                      DNS to approved                   Hostnames, TLS,                 Record successful

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                        NETWORK SETUP & LIVE STREAMING

WHAT                                WHY                                HOW                                 PROVE IT

resolver(s); time                  schedules and                       resolution and device

synchronisation to                 authentication depend on            clock/timezone.

approved source(s).                them.

Live streaming                      Site-specific only: exact          Streaming transport                 Play the actual live

HLS/DASH/RTMPS/SRT/RT              requirements vary; no               channel on target

SP-RTP/WebRTC or                   universal live-stream UDP           hardware for the agreed

multicast rules selected           range exists.                       soak period.

for the implementation.

Display control                     Site-specific only:                Power/input/volume                  Test connection and one

controller-to-display LAN          controls, where                     approved command on

ports for supported                supported, are separate             the physical display.

manufacturer adapters.             from cloud Player HTTPS.

Inbound internet to                 None for normal Dhruv              Normal Player operation             External scan confirms no

Players                             signage unless a                   is device-initiated;                unintended Player

separately documented              unsolicited WAN                     listener is exposed.

service explicitly requires        exposure is unnecessary.

it.

The minimum IT handover

Record the site, player models, installed firmware/app builds, device VLAN/subnet, DNS and time servers,

CMS/player base URLs, media hosts, stream source and transport, authentication owner and maximum

simultaneous screens. Missing values stay TBC, not assumed defaults.

Three rules that prevent most configuration errors

Direction matters. “Outbound TCP 443” means the site device initiates a connection to a destination listening on

443. It does not mean forwarding 443 from the internet to the display. Stateful return traffic normally follows the

established session. [S11]

A protocol default is not a deployment instruction. SRT needs a configured UDP port; RTP media ports are

negotiated or configured; HTTPS on a non-standard port still requires that exact port. [S4–S6]

Open does not mean compatible. Codec, DRM, firmware, app capabilities and authorisation must also pass. Keep

unsupported SoC controls unavailable; do not expose debug ports to make a feature appear supported. [R3, R6]

Sources are identified as [R…] for repository evidence and [S…] for external primary documentation. Full references appear in sections 22–24.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

02 / NETWORK SETUP

Evidence, scope and reader’s map

Repository evidence establishes what the inspected files say. It does not identify the running production build or

prove any physical installation.

Item                     Evidence at preparation                               What remains to be verified

Pinned commit 9a372468f736; full identifier on the    Which image/revision is running on Hetzner and

Dhruv main

cover. [R1]                                           each device.

cms.dhruv.org.uk and player.dhruv.org.uk are          Live DNS, certificate, reverse-proxy mounts and

CMS / Player targets

named in the source material. [R3, R5]                authorised responses.

Actual production networking and absence of

Internal API             Dashboard Nginx points to api:3001. [R2]

public exposure.

An independent hotel IPTV/H200 workstream exists.     Its public host, enabled routes, firmware transport

Amino service

[R6]                                                  and real H200 acceptance.

Planning context: existing Hetzner server described   Actual free space, CPU/RAM, network allowance,

Hosting capacity

as about 80 GB; Mac Docker for local work.            load and recovery evidence.

Status language: Repository-confirmed = inspected source. Vendor-documented = external product requirement. Example = replace

before use. Site TBC = not yet supplied/verified. No item is marked live-verified, hardware-accepted or USER VERIFIED by this

document.

Reader’s map

Sections               Use

03–05                  Survey the site, understand the connections and identify destination hosts.

06–08                  Check Dhruv API paths and prepare baseline/server firewall rules.

09–13                  Choose streaming transports, multicast, WebRTC and Amino requirements.

14–17                  Approve support access, hosting, TLS/proxy behaviour and capacity.

18–21                  Run diagnostics, commission, troubleshoot and obtain sign-off.

22–24                  Open the pinned repository and primary technical references.

This document is network-focused. It does not grant content rights, device compatibility, licences, cloud access or

permission to publish a release. Optional EPOS, ESL, CRM, booking, AI and emergency integrations require their own

provider endpoint records and acceptance tests.

Key terms

SoC: the screen’s built-in computing platform. FQDN: a complete hostname. VLAN: a logical network segment. DNS: hostname

resolution. DHCP: automatic address assignment. NTP: network time. TLS: authenticated encrypted transport. CDN: distributed

content delivery. NAT: network address translation. DRM: content-rights enforcement. TBC: to be confirmed.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                      NETWORK SETUP & LIVE STREAMING

03 / NETWORK SETUP

Site prerequisites and survey

Complete this survey before equipment arrives. Use the final network, not a guest Wi-Fi connection or an installer’s

mobile hotspot, for acceptance.

Area                       Required information / preparation

Site name; IT, AV and Dhruv owners; approved change window; emergency contact; remote-access

Ownership

approver.

Manufacturer, exact model, MAC address, firmware, player/app build, screen location, power mode

Devices

and wired/Wi-Fi capability.

Player VLAN/subnet, gateway, DHCP scope/reservations, DNS resolvers, NTP servers, IPv4/IPv6

Network

policy, MTU and routing.

802.1X/NAC or approved MAC registration; unattended connectivity; no interactive captive portal.

Access policy

Confirm device support before choosing authentication.

Tested switch ports and cabling, negotiated link speed, suitable power supply or verified PoE

Cabling / power

budget, UPS requirements and labelled ports.

Approved CMS/player FQDNs, valid certificates, reverse-proxy owner, service identities and secure

Service identity

enrolment delivery.

Source, destination, transport, codec, resolution, frame rate, peak bitrate, audio, DRM, latency

Streaming

requirement and simultaneous receivers.

Monitoring, log retention, certificate renewal, storage alerts, patching, backups and a tested

Operations

restore/rollback owner.

Recommended logical separation

Use separate policy groups or VLANs for players, encoders/headend, servers and privileged management.

Guest/student/public-client networks should not initiate connections to display management or backend services.

A kiosk’s public user interface does not justify access to the entire corporate LAN.

Allow player-to-player or controller-to-player traffic only for an explicitly implemented feature. Normal cloud

content polling does not require a flat LAN. Where multicast is selected, design the multicast domain rather than

simply disabling segmentation. [R3, S7]

Addressing and time

Prefer managed reservations for headends, controllers and devices requiring inbound LAN control. Record real

addresses in the handover; do not copy illustrative subnets from this guide. Keep the selected time source

reachable and check the displayed timezone/schedule as well as UTC clock accuracy.

Proposed baseline: wired connectivity for fixed live-streaming receivers; Wi-Fi requires a site survey and load test. Do not treat a

good signal indicator as evidence of sustained streaming capacity.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

04 / NETWORK SETUP

Connection architecture

Separate content management, media transport and device management. These may use different hosts and ports

even when presented through one Dhruv interface.

Control and content flow

Initiator                    Destination                            Transport / purpose

Administrator browser        Approved CMS HTTPS edge                TCP 443: sign-in, configuration, upload and publish.

Internal dashboard HTTP 80; Core at api:3001 in inspected

HTTPS edge / dashboard       Internal dashboard and Core API

Nginx. [R2]

Configured database connection; PostgreSQL commonly TCP

Core service                 Approved private database

5432 with production TLS. [R5, S1]

Device-initiated enrolment exchange, content polling, asset

Player / SoC app             Approved Player HTTPS edge

download and heartbeat. [R3–R5]

Only when the assigned content uses an external origin; its

Player / browser             Approved additional media host

auth and host rules must be recorded.

Live streaming flow

Camera / encoder

-> configured ingest endpoint (for example SRT or RTMPS)

-> media server / packager / authorised provider

-> playback endpoint (for example HTTPS HLS)

-> approved decoder or SoC application

-> display

The encoder’s ingest protocol need not be the player’s playback protocol. A site streaming SRT to a provider can

receive HLS over HTTPS without opening SRT to every screen. The CMS must not be assumed to transcode, relay or

store live video merely because it holds the stream URL. [S3, S4]

Local and management paths

For LAN IPTV, the headend sends to the allocated multicast group; receivers join the channel through the

switching/routing infrastructure. For manufacturer management, an H200 may separately contact Amino

Orchestrate. Neither path is replaced by the Dhruv CMS firewall rule. [S7, S13]

Normal signage design: no unsolicited WAN-to-screen connections. A deliberately selected site-hosted streaming listener or LAN

control adapter is an exception requiring an explicit owner, source restriction, authentication and acceptance test.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

05 / NETWORK SETUP

Destination host register

These are the starting records for the allowlist. A firewall matches destination hosts/IPs and ports; an HTTP-aware

proxy can additionally match paths. Do not confuse the two.

Destination                               Role / port                            Status and boundary

CMS browser origin; planned TCP        Repository-named target. Confirm deployment,

https://cms.dhruv.org.uk

443.                                   DNS, TLS and routes before approval. [R5]

Source-configured target, not a live test result.

https://player.dhruv.org.uk               Android Player base URL; TCP 443.

[R3]

Exact hosts returned by the            Site TBC. Inventory manifest, segments, redirects,

Media / object storage / CDN              approved content or stream             keys and licences; avoid unrestricted wildcard

service; usually HTTPS.                egress.

Approved sign-in/discovery/token       Conditional. Administrator/browser or server

Identity provider                         endpoints for the configured SSO       traffic as actually required; not a blanket Player

tenant.                                dependency.

The authority used by the

Site TBC. Do not invent licence.dhruv.org.uk or

Licence authority                         deployed product entitlement

assume direct device access.

service.

Approved Orchestrate or self-          Conditional; see section 13. Do not assume the

H200 configuration / lineup

hosted provisioning origin.            hotel service shares the Core API host.

Exact staging, relay and third-

Site TBC; separate identities and rules from

Staging / support / integrations          party hosts approved for this

production.

installation.

Installation URLs are not yet verified

The repository handover describes target shapes https://player.dhruv.org.uk/install/lg and

https://player.dhruv.org.uk/install/samsung. Treat these as installation design targets, not confirmed download

services or proof that any model is supported. [R9]

How to maintain the register

For every destination, record the FQDN, resolved addresses, destination port, protocol, initiating component,

authentication, owner and date tested. Use provider-managed hostname rules where available. An IP-only firewall

needs an approved address-maintenance process; one DNS lookup is not a permanent CDN IP list.

A redirect introduces another destination. Test the complete content journey, including browser resources and

signed URL expiry. Keep media URLs without secrets in the shared register; store credentials and unredacted signed

links in approved secret storage.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                        NETWORK SETUP & LIVE STREAMING

06 / NETWORK SETUP

PROVE IT: Validate the deployed release: unauthorised calls fail correctly; an enrolled Player can retrieve only its

authorised content and report heartbeat/status.

HOW: Map the repository-defined paths to the deployed HTTPS origin. Preserve authentication headers and do not

expose private database or container ports.

WHY: IT teams often need both the hostname/port and the application path to diagnose proxies, WAFs,

authentication and content delivery.

WHAT: The HTTP/API paths used by the CMS and enrolled Players.

Dhruv application endpoints

Paths below are repository-documented contracts. Validate their public host mapping against the running release;

the same path can be unavailable on a different gateway or build.

Method and path                                                            Caller / purpose

CMS session validation; also used by the internal

GET /api/auth/me

dashboard auth subrequest. [R2]

GET /api/core/devices                                                      Authorised CMS administration: device inventory. [R5]

Privileged CMS enrolment creation; returns a protected

POST /api/core/device-enrolments/bootstrap

one-use sidecar. Not a network smoke test. [R5]

Device-side exchange of the authorised one-use

POST /api/device-enrolments/exchange

enrolment material. [R5]

Device credential verification. Do not supply a shared

POST /api/device-credentials/verify

fleet credential. [R5]

Assigned content manifest for that authenticated

GET /api/players/:deviceId/content

device. [R4]

GET /api/player/assets/:assetId                                            Governed content delivery to an authorised device. [R4]

POST /api/players/heartbeat/:deviceId                                      Authenticated device status/heartbeat. [R4]

Authorised CMS screen status in the permitted scope.

GET /api/core/screens/:screenId/status

[R4]

Authorised publishing with an Idempotency-Key. This

POST /api/core/schedules/:scheduleId/publish

changes state. [R4]

Site-scoped service identity with player:authorize; not

POST /api/player/authorization

anonymous public access. [R4]

Browser and gateway paths

The inspected dashboard serves /cms/ behind session validation, forwards /api/ and /uploads/ to Core, and

keeps /_dhruv_cms_auth internal. A generic browser password gate must not inadvertently challenge device,

media or webhook requests. [R2, R8]

A route existing in source is not proof of deployment. Do not invent a /health endpoint, command-poll path or WebSocket URL.

Obtain the running service’s route contract before writing path-specific WAF rules. A 200 response containing the SPA’s index.html

is not a successful API test.

Endpoint rules must preserve tenant/site/device authorisation. Network allowlisting never replaces authentication. Test only

authorised disposable identities for negative and enrolment tests; never paste production tokens into shared command

examples.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                        NETWORK SETUP & LIVE STREAMING

07 / NETWORK SETUP

PROVE IT: Test each rule from the same VLAN and device class that will run Dhruv. Record hostname, destination

port, result and date.

HOW: Start with DNS/time and outbound HTTPS. Add external content, live-stream or hardware-control rules only

when the feature is selected. Keep stateful return traffic rather than opening inbound WAN rules to Players.

WHY: Dhruv needs connectivity to named services, but broad internet or east-west access increases risk without

improving normal playback.

WHAT: A least-privilege set of rules for the Player VLAN and any server/management VLANs.

Baseline site firewall matrix

Allow only the applicable rows. Ports shown are destination ports unless the source port is explicitly stated. Use

established/stateful return traffic rather than unrestricted inbound ephemeral ranges.

Rule      Initiator → destination                            Protocol / port                    When / restriction

B01       Players → approved Player service                  TCP 443                            Core HTTPS profile; exact approved host.

CMS use; restrict administration as site

B02       Admin clients → approved CMS service               TCP 443

policy requires.

Only hosts actually needed by assigned

B03       Players / clients → approved media origins         TCP 443

content.

Name resolution; not all internet DNS

B04       Devices / servers → approved DNS resolvers         UDP and TCP 53

servers.

B05       Devices / servers → approved time servers          UDP 123                            Time synchronisation where NTP is used.

Local address service; not a WAN port-

B06       DHCPv4 client ↔ server / relay                     UDP 68 → 67; replies 67 → 68

forward.

Only when DHCPv6 is used. IPv6 also

B07       DHCPv6 client ↔ server / relay                     UDP 546 / 547                      needs an approved ICMPv6/RA/ND

policy.

Exception only; plain HTTP is not the

B08       Clients → specified legacy / probe host            TCP 80

Android Player baseline.

Approved clients → configured HTTP/3                                                  Optional QUIC/HTTP/3, not required for

B09                                                          UDP 443

service                                                                               ordinary TCP HTTPS. [S21]

Approved management source → VPN /                 Configured VPN transport /         Separate approved design; no guessed

B10

bastion                                            port                               universal VPN port.

Basis: Dhruv HTTPS source and standard service assignments. IPv6 policy must be designed separately rather than copying IPv4 rules. [R3, S1,

S17]

Default restrictions

Deny unsolicited internet-to-player traffic. Deny player access to unrelated corporate, guest and management

destinations. Do not publish ADB, database, development servers, screen control APIs or remote desktop ports to

the internet. Apply equivalent policy to both address families.

What “required” means here

DNS, address and time services may be supplied locally. Devices using static addresses do not need a new DHCP

rule merely because it is listed. A selected integration can require additional egress, but that is approved against its

exact service record, not by opening every optional row in this guide.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

Keep essential ICMP / ICMPv6 error handling available under site policy so MTU discovery and IPv6 functions are not broken. “Deny

all ICMP” is not a safe substitute for a designed firewall policy. [S17]

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

08 / NETWORK SETUP

PROVE IT: Scan from outside the host/network: only approved public services should answer. Confirm private ports

are unreachable externally and the HTTPS route reaches the correct application health/readiness path.

HOW: Expose only the authorised HTTPS edge publicly. Bind application and database services privately or to

loopback/internal networks unless a documented architecture requires otherwise.

WHY: Publishing an internal service port directly bypasses the intended TLS, authentication, logging and rate-

control edge.

WHAT: The separation between public HTTPS ingress and private application/database/container services.

Hosted edge and private service ports

External clients should reach the approved HTTPS edge, not the database or application container directly. Match

these rules to the actual deployed composition.

Traffic / service                         Port / transport                     Exposure policy

Public Player origin as designed; CMS

Client → public HTTPS edge                TCP 443

administration can be further restricted.

Only if that certificate/redirect design needs it.

ACME HTTP-01 / approved redirect          TCP 80

DNS-01 does not require inbound 80. [S10]

TCP 22, unless deliberately          Approved VPN/bastion or tightly scoped sources;

Admin → SSH host

changed                              key-based access and no public password login.

Observed dashboard listen port; not a

Edge → dashboard                          Internal TCP 80

requirement to expose raw HTTP publicly. [R2]

api:3001 in inspected Nginx; deny direct public

Dashboard → Core API                      Internal TCP 3001

access. [R2]

Configured port; commonly TCP        Private app-to-database only. Production store

Core → PostgreSQL

5432                                 requires database TLS. [R5, S1]

Container TCP 3000 in local          Separate workstream. Do not reuse public 3000 as

Independent H200 service

Compose                              the production API design. [R6, R7]

Only if the presentation module is actually

Optional presentation controller          Internal TCP 8092

deployed. Not needed by standard screens. [R2]

Actual approved destinations and     Server-originated; do not grant players the

Monitoring / backup / update traffic

ports                                server’s maintenance access.

Container publication is a separate security boundary

A Docker port published on all interfaces can be reachable even when the application is considered “internal”.

Docker also manages firewall rules, and its published traffic can bypass the usual UFW path. Review the actual

Docker firewall backend, provider firewall and host rules together. [S2]

Prefer no host publication for private services. Where local diagnostic access is justified, bind to loopback, for

example 127.0.0.1:<host-port>:<container-port>, and verify the resulting socket bindings. Do not run a generic

firewall script from this document against an existing production host.

Opening inbound 443 on Hetzner and opening outbound 443 at the customer site are different changes. The first permits access to

the server; the second permits a device to initiate a request to it.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                   NETWORK SETUP & LIVE STREAMING

09 / NETWORK SETUP

PROVE IT: Test the complete encoder-to-platform-to-screen journey, including authentication, codec support,

reconnect and sustained playback.

HOW: Choose the actual stream architecture first, then approve its exact listener, direction, destination and media

range. Never use a generic high-UDP range unless the configured service genuinely requires it.

WHY: “Live streaming” is not one port. HLS over HTTPS, SRT, RTSP/RTP, multicast and WebRTC have different

connection and firewall behaviour.

WHAT: A protocol selection table for live contribution and playback.

Live-streaming port matrix

Choose the transport from the actual ingest or playback URL and device configuration. This table describes protocol

behaviour, not universal support by all Dhruv players or SoC models.

Transport                  Typical listener / destination                    Required network decision

Player → playback origin/CDN. Include manifests,

HLS / DASH over HTTPS      TCP 443; custom port if specified

segments and required key/DRM destinations. [S3]

Legacy/provider exception only; do not disable

HTTP playback              TCP 80 or configured port

Android TLS protections to use it. [R3]

RTMP commonly TCP 1935; RTMPS uses the            Encoder → ingest listener. Do not assume a universal

RTMP / RTMPS ingest

provider-configured TLS port                      RTMPS port or open 1935 to every player. [S1]

RTSP TCP 554; RTSPS TCP 322; 8554 is an           Client → RTSP server. Confirm transport/version on

RTSP / RTSPS control

alternative service setting                       both endpoints. [S5]

Negotiated/configured UDP ports; or RTSP          Record the exact range and both endpoint roles. Port

RTP / RTCP media

TCP-interleaved media                             554 alone may not carry video. [S5]

Record listener, caller, NAT mapping and media

SRT                        Configured UDP port; 9000 is only an example

direction. No fixed universal SRT port. [S4]

Assigned group address and UDP destination        Headend → joined LAN receivers; IGMP is separate

UDP / RTP multicast IPTV

port                                              control traffic. [S7, S8]

Application-specific media; TURN defaults         Record ICE servers, relay range, signalling and

WebRTC / TURN

3478 / 5349                                       fallback. See section 12. [S6]

Optional HTTPS transport optimisation; test

HTTP/3 / QUIC              UDP 443 when enabled

permitted fallback to TCP. [S21]

Do not approve guessed media ranges

Ranges such as “all UDP high ports” or “10000–20000” are not generic Dhruv requirements. Use the range

configured on the actual RTP receiver, media server or TURN server, then test both normal operation and

reconnection. A range from a different vendor’s example is not evidence for this installation.

A hostname and open port are insufficient to define a stream. Record channel/path, protocol, codec, bitrate,

latency, encryption, authentication, source restrictions and failover behaviour. Validate content rights and the

selected application’s playback capability before commissioning.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

10 / NETWORK SETUP

Setting up unicast live streams

Keep contribution (encoder to platform) separate from distribution (platform to screens). Approve one complete

end-to-end path before adding more channels or devices.

HTTPS HLS / DASH playback

Create a stream record using an approved playback URL. HLS playlists can refer to further playlists, media

segments, initialisation data and encryption keys; those destinations also need to be reachable. For a browser

player, check cross-origin policy and mixed-content restrictions. [S3]

Example only — not a Dhruv endpoint:

https://stream.example.com/live/channel01/index.m3u8

https://stream.example.com/live/channel01/manifest.mpd

Test with the intended device and credentials, including redirects and URL renewal. Confirm that a live playlist

advances and actual audio/video play for the agreed soak period. A successful request for the first playlist is not a

streaming acceptance result. Set stream-loss fallback and alerting; cached signage does not cache a continuing live

event.

RTSP / RTP

Agree whether media use negotiated UDP ports or TCP interleaving. With UDP, record the receiver range, source

and return/control behaviour; with interleaving, verify that both endpoints support it. Do not assume a proxy will

automatically handle RTSP negotiation or NAT. Keep unauthenticated camera/encoder services off public networks.

[S5]

SRT

Set one endpoint’s connection role and the other’s matching role. Caller/listener identifies who establishes the

connection, not who sends the video. An outbound caller at the site can simplify firewall traversal, but the receiving

listener still needs a reachable configured UDP socket. [S4, S19]

Example contribution path only:

Encoder (caller) -> ingest.example.com : UDP 9000

Listener -> established return traffic to encoder

Player -> approved HTTPS playback origin : TCP 443

Record the actual port, stream ID, encryption policy, latency and NAT/public address. Keep passphrases outside

URLs in shared documentation. Test packet loss, reconnect and the selected latency under representative load. A

TCP port test does not verify SRT. If the chosen listener is behind carrier-grade NAT, obtain a supported

relay/outbound design rather than promising an ordinary port-forward. [S19]

Do not route SRT, raw RTMP or TURN through an ordinary HTTP reverse-proxy location. They need transport-appropriate listeners

or gateways. The current Dhruv dashboard’s /api/ proxy does not establish a streaming server. [R2]

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                         NETWORK SETUP & LIVE STREAMING

11 / NETWORK SETUP

PROVE IT: Join/leave channels on real receivers, verify switch group tables, confirm no flooding to unrelated ports,

and test multiple simultaneous receivers.

HOW: Use an IPTV VLAN, IGMP snooping, a querier where required, controlled routing/PIM only when designed,

and explicitly allocated multicast groups/UDP ports.

WHY: Multicast can distribute one stream efficiently to many receivers, but unmanaged flooding can disrupt a

signage or corporate LAN.

WHAT: Local one-to-many IP video using multicast groups and multicast-aware switching.

LAN IPTV and multicast

Use multicast only when the selected headend, switches, routing and receivers have been designed and tested

together. It is not a public-internet distribution method.

Channel allocation example

Record                              Illustrative value                      Replace with

Headend source                      10.20.30.10                             Approved source address and interface

Multicast group                     239.10.10.10                            Site-assigned administratively scoped group

Destination port                    UDP 5000                                Actual channel port from encoder/lineup

Receiver network                    AV player VLAN                          Actual VLAN/subnet and switch ports

TTL / hop scope                     Set for the approved path               Documented headend and routing setting

Join mode                           IGMP version agreed by design           Version and source-specific support of all devices

All addresses and ports in this example are illustrative. Administratively scoped IPv4 multicast addressing is defined in RFC 2365. [S8]

Switching and routing checklist

Enable and verify IGMP snooping on receiver VLANs and ensure there is a working elected querier. Check receiver

membership, channel changes and group ageing. Do not enable fast-leave on a port carrying multiple downstream

receivers without a validated design. [S7, S20]

IGMP is IP protocol 2, not TCP or UDP port 2. It manages IPv4 membership; the channel payload uses its assigned

UDP port. Cross-VLAN multicast needs explicit multicast routing and scope/ACL policy, not merely ordinary unicast

routing. IPv6 multicast requires its own MLD/ICMPv6 design. [S7, S17, S18]

Limit source, group, receiver VLANs and routed scope. Avoid flooding multicast to guest Wi-Fi, unrelated users or

all switch ports. A single channel can consume bandwidth on every forwarding uplink; verify shared trunks and

downstream fan-out, not only the headend NIC.

Acceptance evidence

Record group membership and counters, verify every receiver displays the correct channel, change channels

repeatedly, restart one receiver, and test after the agreed membership-ageing interval. Check that non-member

ports are not unexpectedly flooded and that a querier failure is detected. Record packet loss, visible artefacts and

recovery under load.

Wi-Fi multicast behaviour varies by access-point configuration. Use a validated multicast-to-unicast or other supported design

where required; do not assume an Ethernet success proves a wireless installation.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

12 / NETWORK SETUP

WebRTC, timing and multi-screen playback

These are conditional network profiles. The repository’s ordinary HTTPS polling does not by itself prove that a

WebRTC or frame-synchronised playback path is deployed.

WebRTC connection inventory

Layer                           Record before opening rules

Signalling                      Actual HTTPS/WSS hostname, path, port and authentication.

ICE discovery                   Exact STUN/TURN hostnames, URLs, transport and credentials policy.

Common defaults: UDP/TCP 3478; TLS-over-TCP or DTLS-over-UDP 5349. Use the deployed

TURN access

configuration. [S6]

Media / relay                   Actual SFU/media listener and configured relay allocation range; peer/source policy.

Restricted-network fallback     A deliberately configured TURN-over-TLS path, sometimes on 443; forced-relay test required.

Maximum sessions, codecs, bitrate and server relay egress; do not size on signalling traffic

Capacity

alone.

HTTPS/WSS connectivity does not prove media connectivity. TURN also allocates relay addresses/ports for the peer

path. Do not assume that opening 3478 alone completes every TURN firewall path, or that any HTTPS server on 443

can accept TURN. [S6]

When placing TURN TLS and HTTPS on port 443, provide separate addresses or a documented compatible

transport-layer design. Keep the relay range constrained to the server’s actual configuration. Test direct and

forced-relay calls from the final site network, including restrictive egress conditions.

Screen groups and synchronised content

Provide reliable time service, consistent device timezone/schedule settings, sufficient download capacity and

preloading where supported. But NTP, shared Wi-Fi or identical playlists do not establish frame-accurate

synchronisation. The inspected Android source explicitly separates readiness preparation from physical

synchronised-start evidence. [R3]

For a multi-screen splash, record the supported player/runtime, control path and any actual local timing transport.

Do not open PTP, broadcast or peer-to-peer ports unless the selected implementation uses them. Test all screens

together and measure start skew, drift, recovery and fallback against an agreed tolerance.

Keep a working ordinary signage page available when an optional control or synchronisation capability is unsupported. Missing

model-specific support is not a reason to open every management port or disable authorisation.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

13 / NETWORK SETUP

Amino H200 setup and cloud ports

Select the provisioning method first. H200 can use Orchestrate, a self-hosted configuration service or USB

provisioning; a self-hosted installation does not automatically require all Orchestrate destinations. [S14]

Hardware and application prerequisites

Record the H200/H200W model, firmware and application, Ethernet/HDMI path, power arrangement,

configuration/lineup origin and approved playback transport. Amino documents gigabit Ethernet and model-

dependent PoE/Wi-Fi options. Verify the actual unit and switch power budget. The Dhruv H200 workstream still

requires firmware transport confirmation and real-device acceptance. [S14, R6]

Orchestrate destination matrix — conditional

Vendor-published requirements, updated 2 April 2026. These are device-originated management connections, not general

signage ports. Confirm the assigned region and current vendor policy before applying. [S13]

Destination                                                             Destination port / transport

connectivitycheck.gstatic.com                                           TCP 80 — connectivity probe

www.google.com                                                          TCP 443 — connectivity probe

orch-acs.aminoengage.com                                                TCP 7743 — HTTPS configuration / TR-069

ensight-cmd.aminoengage.com                                             TCP 443 — command redirection

orch-logfile.aminoengage.com                                            TCP 10446 — HTTPS log upload

gcs.amino-orchestrate.com                                               Vendor table groups TCP 80 / 443 for firmware;

files.amino-orchestrate.com                                             confirm exact host/URL pairing.

orch-stun.aminoengage.com                                               UDP 3478 — management STUN

EMEA Resolve cluster:

ensight-cmd31.aminoengage.com

TCP 22, TCP 10444 and UDP 1194

ensight-cmd32.aminoengage.com

Only for the applicable device-access service/region.

ensight-cmd33.aminoengage.com

ensight-cmd34.aminoengage.com

Do not open both US and EMEA clusters by assumption. Reconfirm DNS/IP requirements with Amino; resolve the firmware table’s

grouped entries before narrowing production rules. Cloud management STUN is not evidence that Dhruv playback uses WebRTC. [S13]

Local H200 control is a separate record

For an enabled local EELM/vendor API, record the exact firmware manual, transport, listening port, authentication

and controller source. None is treated as a universally verified local control port here. Channel media follows the

selected streaming profile; cloud management ports do not replace it.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

14 / NETWORK SETUP

Display control and remote support

Use the normal authenticated device channel for supported controls. Local control and remote support need their

own approval, access scope and audit trail.

SoC and panel control

Do not assign one universal Samsung, LG, SOLUM or Android management port. Exact model, firmware, enabled

API and installed adapter determine the transport. A controller on the LAN may need a device listener; another

model may use only device-initiated HTTPS. RS-232 and HDMI-CEC do not themselves require TCP/UDP firewall

ports.

Keep unsupported power/input/brightness functions unavailable rather than enabling a debug service to simulate

support. Check whether the panel’s network interface remains reachable in standby before promising remote

power-on. Record Wake-on-LAN or vendor wake behaviour only when explicitly supported and tested.

Temporary engineering access

Legacy Android ADB over TCP commonly uses 5555; modern wireless debugging uses a pairing/connection

workflow with negotiated ports. Neither is required for ordinary Dhruv operation. Restrict any approved temporary

use to a management host/VLAN, then disable it and re-test isolation. Never expose it publicly. [S9]

Optional RustDesk self-hosted profile

Service                       Destination ports                           Approval boundary

Clients → approved ID server; server listener/firewall

ID / connection / NAT test    TCP 21115; TCP and UDP 21116

must match.

Clients → approved relay; test forced relay and

Relay                         TCP 21117

bandwidth.

Keep closed if unused. If used, restrict to the trusted

Web client only               TCP 21118 / 21119 internally

reverse proxy.

Not an OSS baseline requirement; restrict administration

Pro console only              TCP 21114

separately.

The official RustDesk documentation warns that direct access to its WebSocket listeners permits spoofed

forwarding headers. Put those listeners behind a correctly configured trusted proxy when used. No RustDesk server

or hostname is confirmed deployed by this guide. [S15, S16]

Prefer approved relay-only support where direct peer connectivity is not permitted; test that mode rather than allowing arbitrary

destinations. Remote-control permissions, unattended access, server identity keys and operator logging are separate from opening

the server ports.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

15 / NETWORK SETUP

Hetzner hosting and Mac Docker

Use the existing server only after confirming its current composition and capacity. Local development, hosted

staging and production must remain separate security and data boundaries.

Hosted setup sequence

1. Inventory and back up. Record running services/images, compose/launch files, mounts, certificate management,

listening sockets, DNS, free disk, private networks and rollback route. Capture configuration without exposing

environment secrets.

2. Choose service origins. Confirm the production CMS/player hosts and create a separately approved staging

hostname only when DNS, TLS and the owner are available. A placeholder such as api-staging.example.com is not a

provisioned service.

3. Isolate deployments. Use separate databases/volumes, credentials, ports and networks. Do not let staging enrol

devices against production or reuse its signing/licensing secrets. Expose the approved HTTPS edge only; keep

service listeners private.

4. Verify all firewall layers. Hetzner Cloud Firewalls are stateful; once outbound rules are added, unmatched

outbound traffic is denied. The retrieved FAQ says they do not secure private Cloud Network traffic, so enforce

appropriate host/container controls too. Confirm this applies to the actual Hetzner product in use. [S11]

5. Prove the running route. From the final client network, test DNS/TLS, authenticated API access and content

delivery. From an authorised external test host, verify private backend/database ports are unreachable. Preserve

evidence before widening access.

Mac Docker development

Identify the maintained workstream’s actual compose file rather than assuming a root-level docker-compose.yml

exists. Prefer loopback-only publication for developer tools. The independent H200 example maps 3000:3000;

harden the effective development binding where LAN access is not needed. [R7, S2]

Illustrative port-binding fragment only:

ports:

- "127.0.0.1:3000:3000"

# Use the correct service and reviewed override for the chosen stack.

A phone or screen’s localhost is that device, not the Mac. To test a physical device, use an approved reachable

Mac/LAN address and the application’s supported trusted-TLS configuration, with source-restricted host firewall

access. Keep production credentials out of local files and shared screenshots.

The described 80 GB disk and one-to-two-connection staging workload are planning inputs, not proof of streaming, transcoding or

production capacity. Measure storage and bandwidth; keep backups outside the single host and test restoration.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                   NETWORK SETUP & LIVE STREAMING

16 / NETWORK SETUP

DNS, TLS, proxies and authentication

Connectivity must preserve the application’s security model. Do not “fix” a certificate, proxy or authentication error

by disabling validation or sharing one device credential.

Topic                     Required setup / validation

Correct A/AAAA records, gateway and resolver reachability; stable approved FQDNs. Avoid raw-IP URLs

DNS / identity

when the certificate and auth origin expect a hostname.

Use a valid hostname-matching certificate and complete chain. Test device trust stores and clock. The

TLS trust

Android source uses system trust anchors and disables cleartext. [R3]

Document the site inspection policy. Use an explicitly approved endpoint exception if interception is

TLS inspection

incompatible; do not set “trust all” or use curl -k as acceptance.

Confirm unattended device proxy support. Exclude interactive captive portals and browser-only

Proxy / content filters

authentication challenges from device paths.

The Android source attaches device credentials only to its exact configured HTTPS origin. Use an

Cross-host media

approved signed URL/auth mechanism for other origins; never forward the bearer token blindly. [R3]

Permit the actual route methods. Test authorised uploads, media byte-range delivery when required

Methods / downloads

and signed URL renewal; do not cache personalised API responses.

A browser media/widget may need approved origins, redirects and subresources. An open TCP port

CORS / embedding

does not override browser CORS, CSP or mixed-content checks.

Record renewal owner and challenge. HTTP-01 needs TCP 80; DNS-01 can avoid inbound 80; TLS-

Certificates / renewal

ALPN-01 uses 443 with supporting infrastructure. [S10]

Known reverse-proxy review point

The inspected dashboard API proxy has an 8-second read timeout and WebSocket upgrade headers. That does not

prove a long-lived socket/SSE or streaming endpoint exists. For an implemented long-lived route, review timeout,

buffering and heartbeat behaviour specifically; do not relax every API route indiscriminately. [R2, S12]

Keep /api/ handling ahead of the single-page application fallback. Preserve the trusted forwarding chain and

session/device distinction. Do not apply a browser Basic Auth prompt to machine device endpoints, and do not

allow a service token to bypass a browser-only administration gate. [R2, R8]

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                     NETWORK SETUP & LIVE STREAMING

17 / NETWORK SETUP

Capacity, bandwidth and continuity

The figures below are worked planning examples, not measured Dhruv performance or minimum hardware

guarantees. Replace every bitrate, concurrency and headroom assumption with the selected service’s data.

Bandwidth calculations

Scenario / assumption                                   Calculation                           Planning result

20 unicast screens, each 8 Mb/s; 30% headroom           20 × 8 × 1.30                         208 Mb/s site downstream

One 8 Mb/s outgoing encoder; 30% headroom               8 × 1.30                              10.4 Mb/s site upstream

50 unicast screens, each 6 Mb/s; 30% headroom           50 × 6 × 1.30                         390 Mb/s site downstream

3.6 GB per receiver-hour before

One 8 Mb/s stream for one hour; decimal units           8 × 3,600 ÷ 8 ÷ 1,000

overhead

177.8 Mb/s average before

20 devices each fetch 1 GB in 15 minutes                20 × 1 × 8,000 ÷ 900

overhead

These calculations treat the stated bitrate as the total selected stream payload. Add audio separately if it was

excluded, and account for peaks, retransmission, adaptive variants, background downloads, VPN overhead and

shared business traffic. The 30% factor is an illustrative design allowance, not a universal engineering standard.

Distribution and storage

Unicast generally scales with simultaneous receivers. Multicast forwarding depends on channels and membership

on each link; a shared uplink may carry one copy per channel while each receiver link still carries its selected

stream. Verify the real topology and switch counters. [S7]

The CMS need not carry every video byte if an approved origin/CDN supplies media. Conversely, a self-hosted

relay/packager adds its own egress, CPU and storage load. Do not estimate transcoding capacity from disk size. Set

separate capacity budgets for database, media, logs, images and backups.

Continuity policy

Set download windows and staged rollouts so a full fleet does not update simultaneously. Define which cached

content remains eligible during outages, how an interrupted download is recovered, and what viewers see when a

live stream stops. The Android source validates an offline cache, but this document does not establish hardware

network-loss acceptance. [R3]

Agree recovery-time and data-loss objectives, a stream fallback, alerts, and an observed restart/reconnect

procedure. Test the final number of screens through a representative content cycle and live-stream soak; record

measured latency, start skew, drift, bandwidth and free storage.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                   NETWORK SETUP & LIVE STREAMING

18 / NETWORK SETUP

Network diagnostic commands

Run only on systems and endpoints you are authorised to test. These examples are read-only checks; they were not

executed against your live deployment while preparing this guide.

DNS and TCP reachability

# macOS / Linux — repository-named targets, not live-verified

dig +short cms.dhruv.org.uk A

dig +short cms.dhruv.org.uk AAAA

dig +short player.dhruv.org.uk A

# Windows PowerShell

Resolve-DnsName player.dhruv.org.uk

Test-NetConnection player.dhruv.org.uk -Port 443

TLS and route behaviour

# Certificate chain and hostname verification; never add -k

openssl s_client -connect player.dhruv.org.uk:443 \

-servername player.dhruv.org.uk \

-verify_hostname player.dhruv.org.uk \

-verify_return_error </dev/null

# An unauthenticated route probe: no production token included

curl --connect-timeout 5 --max-time 15 -sS -D - \

-o /dev/null https://cms.dhruv.org.uk/api/auth/me

Record DNS answers, certificate identity/expiry and HTTP status/content type. An unauthorised response can

demonstrate that a route was reached, but does not establish successful sign-in. A login redirect or 200 HTML

response must not be labelled an API pass. Perform the authenticated check through the approved client or secure

test harness.

Read-only host inventory

# Linux host — output may contain sensitive topology

sudo ss -lntup

docker ps --format 'table {{.Names}}\t{{.Ports}}'

df -h

# macOS listening TCP services

lsof -nP -iTCP -sTCP:LISTEN

Verify unexpected 0.0.0.0 / [::] bindings, internal-service exposure and free disk. Use the correct deployed

configuration files, but do not attach environment dumps or secrets to the handover. Test from the actual player

VLAN as well as from an administrator’s laptop.

Ping and TCP tests do not prove UDP media, authentication or playback. Use the actual decoder, server counters and an authorised

bounded packet capture for SRT/RTP/multicast diagnostics. Redact credentials and signed URLs from logs and captures.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                      NETWORK SETUP & LIVE STREAMING

19 / NETWORK SETUP

Commissioning and acceptance checklist

Leave each box unmarked until evidence is retained. “Not applicable” needs a reason and owner. A passed network

test does not automatically pass application, hardware or user acceptance.

Check                    Pass condition / evidence                                                                           Result

01 • Inventory           Models/builds, VLANs, assigned addresses, switch ports and approved endpoints recorded.             []

02 • Address / DNS /

Correct DHCP/static config, A/AAAA resolution, gateway and time; survives reboot.                   []

time

03 • TLS / egress        Valid certificate chain/hostname from real devices; only intended service destinations permitted.   []

04 • CMS session         Authorised user signs in; correct tenant/site; unauthorised requests fail as expected.              []

Disposable approved device enrols once; credential bound to correct device/site; no secret

05 • Enrolment                                                                                                               []

leakage.

Approved test asset is delivered and visibly plays; manifest/asset identity recorded; heartbeat

06 • Publish / content                                                                                                       []

reports it.

Only supported controls shown; actual commands and acknowledgements verified on each

07 • Device controls                                                                                                         []

required model.

08 • Live playback       Correct channel, video/audio/DRM, latency and quality on intended hardware for agreed soak.         []

09 • Selected            RTSP media path, SRT roles/UDP, multicast membership or WebRTC forced relay tested as

[]

transports               applicable.

Power restart, WAN loss/recovery and interrupted download tested; approved fallback and status

10 • Restart / outage                                                                                                        []

truthful.

11 • Isolation /         Private API/DB/debug ports unreachable from unauthorised sources; IPv4/IPv6 and staging

[]

exposure                 separation tested.

Full target concurrency, content refresh, token/certificate lifecycle and storage thresholds

12 • Load / renewal                                                                                                          []

evidenced.

13 • Handover            Owners, firewall change IDs, backups, restore evidence and unresolved issues recorded.              []

14 • User acceptance     Named customer/user signs the actual installation outcome separately from engineering checks.       []

Proposed test record: timestamp/timezone; operator; device ID/model/build; service revision; source VLAN; destination/port;

expected result; actual result; log/screenshot/capture reference; issue/owner. Set the required soak duration and recovery

tolerance before starting, not after seeing the result.

No row above is marked complete by this guide. An unavailable external dependency blocks its specific acceptance test; it does not

justify treating unrelated completed tests as failed or claiming an untested feature passed.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                    NETWORK SETUP & LIVE STREAMING

20 / NETWORK SETUP

Troubleshooting by symptom

Start with the initiating component and its exact destination. Separate network, TLS, identity, media and device

capability failures rather than repeatedly opening more ports.

Symptom                    Likely checks                                       Next evidence

CMS opens; screen stays    Different CMS/player origins; device VLAN           Device request status, DNS answers,

offline                    egress; clock/TLS; enrolment credential.            gateway/firewall logs and authenticated heartbeat.

API returns 200 but data   SPA fallback caught /api/; proxy route              Content-Type and body shape; active proxy

are missing                mismatch; wrong deployed build.                     config/revision. [R2]

Signage works; live        Separate media host/port; redirects/keys;           Full media request chain plus actual decoder error

stream is blank            expired signed URL; unsupported codec/DRM.          and source output.

TLS fails only on the      Device clock; incomplete chain; trust-store age;    Screen clock, verified certificate chain and

screen                     interception; wrong hostname.                       inspection policy. [R3]

RTP/RTCP UDP range or NAT blocked;                  Negotiated transport, source/destination media

RTSP connects; no video

incompatible interleaving.                          sockets and decoder logs. [S5]

Wrong UDP port; two callers/listeners in

Both endpoint roles/configuration and bounded

SRT does not connect       incompatible setup; NAT; stream ID/encryption

UDP capture. [S4]

mismatch.

Multicast stops after a    Missing querier; membership ageing;                 Querier and group membership state over time;

delay                      snooping/routing mismatch.                          port counters. [S7]

Works on laptop, not       Different DNS/proxy/NAC policy; browser trust       Repeat on intended device and VLAN, not through

player VLAN                differs; test device not on final network.          hotspot.

Drops during updates or    Link congestion; Wi-Fi contention; server           Time-aligned traffic, loss, CPU/disk and

peak hours                 egress/storage; shared uplink saturation.           download/stream logs.

Control works when on,     Panel NIC sleeps; unsupported wake/control          Manufacturer-supported standby behaviour and

not in standby             path; model capability mismatch.                    real device test.

H200 plays, but            Management ports/hosts differ from media            Vendor endpoint matrix, device management logs

Orchestrate fails          destinations; wrong assigned region.                and approved rule hits. [S13]

A 502 appears after        Proxy/upstream discovery or readiness; wrong        Actual backend health and resolution; preserve

container replacement      internal port.                                      maintained Docker DNS behaviour. [R2]

Keep a dated problem record and reverse any temporary diagnostic exception after testing. Do not leave public debug ports,

bypassed certificate validation or anonymous media/admin routes behind as a “temporary fix”.

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                       NETWORK SETUP & LIVE STREAMING

21 / NETWORK SETUP

Firewall request and site handover

Copy this page into the installation change record. The final approved rule set is the populated site-specific register,

not every possible port listed in this guide.

Site and service record

Field                                             Approved value

Site / change ticket / change window              To complete

Player VLAN / subnet / gateway                    To complete

DNS / NTP / DHCP or static assignment             To complete

CMS / Player HTTPS origins                        To complete and live-verify

Media hosts / stream URLs (redacted)              To complete

Encoder / receiver models and builds              To complete

Streaming / management profiles selected          To complete; mark unused profiles not applicable

One record for each firewall rule

Field                                             Required entry

Source → destination                              Exact VLAN/host/service group → exact FQDN/IP/group

Transport / destination port                      TCP, UDP or IP protocol; single port or justified configured range

Direction / initiating component                  Site outbound, server inbound, private LAN or multicast; stateful return policy

Reason / authentication / lifetime                Feature, credential owner, permanent or time-limited, expiry/review date

Validation / rollback                             Test ID, evidence, expected rejection test and rollback action

Separate sign-offs

Approval                                          Name / date / evidence

Network rules and isolation                       To complete

Dhruv application / deployed revision             To complete

AV hardware and live-stream acceptance            To complete

Customer / USER VERIFIED                          To complete; never inferred from a merge or successful build

Outstanding dependency / risk: ______________________________

Owner / next action / review date: _____________________________

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                       NETWORK SETUP & LIVE STREAMING

22 / NETWORK SETUP

Repository references

All repository references use the inspected source snapshot, not an assumption that the same revision is deployed.

Some endpoint/Compose entries were established from indexed excerpts rather than a full-file review.

[R1] Repository main snapshot

Commit 9a372468f73656754fdff5f8b83fddd5e6ad8907. Read 16 September 2026; commit timestamp 06:12:42 UTC.

Open source

[R2] Dashboard reverse proxy

Full configuration reviewed: internal listen 80, api:3001, optional presentation-demo:8092, API/upload routing, session gate and 8-

second read timeout.

Open source

[R3] Android signage Player

Full README reviewed: Player HTTPS origin, system trust, credential binding, content cache and explicit hardware/production

acceptance boundaries.

Open source

[R4] Core application route contract

Indexed excerpts reviewed for device content/assets/heartbeat, screen status, publishing and player-authorization route contracts.

Open source

[R5] Device credentials and enrolment

Full contract reviewed: administrative and device-only routes, secure enrolment, PostgreSQL TLS and live-deployment evidence

boundary.

Open source

[R6] Independent hotel IPTV / Amino H200 workstream

Full README reviewed: separate service, scoped APIs, pairing/telemetry and outstanding vendor transport / hardware acceptance.

Open source

[R7] H200 local Compose example

Indexed excerpt reviewed: local port 3000 mapping and PostgreSQL connection. Example credentials intentionally not reproduced.

Open source

[R8] CMS-only perimeter auth snippet

Indexed excerpt reviewed: scope the browser gate to CMS; do not apply it indiscriminately to Player/device/webhook paths.

Open source

[R9] LG and Samsung installation handover

Indexed excerpt reviewed: /install/lg and /install/samsung are target public shapes, not live-verified services.

Open source

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                       NETWORK SETUP & LIVE STREAMING

23 / NETWORK SETUP

Protocol and infrastructure references

Primary technical documentation consulted on 16 September 2026. Service defaults describe protocols;

deployment owners must confirm the configured listener and supported implementation.

[S1] IANA service name and transport port registry

Reference for standard service assignments, including baseline network services. A registry entry does not make a port necessary for

every Dhruv installation.

Open source

[S2] Docker: packet filtering and firewalls

Published-port behaviour, Docker-managed firewall rules and the interaction with UFW.

Open source

[S3] RFC 8216: HTTP Live Streaming

Playlists, media references, keys and associated HLS resources. DASH is listed as a separate selected HTTP media profile, not asserted

as universal player support.

Open source

[S4] Haivision: SRT stream settings

Primary vendor description of SRT caller/listener addressing and configured UDP ports. Product-specific latency defaults are not

adopted as Dhruv requirements.

Open source

[S5] RFC 7826: Real-Time Streaming Protocol 2.0

RTSP/RTSPS defaults and negotiated/interleaved media transport. Confirm the RTSP version and options actually supported by both

devices.

Open source

[S6] RFC 8656: TURN

TURN server transport defaults, relayed addresses and associated peer traffic.

Open source

[S7] RFC 4541: IGMP/MLD snooping considerations

Multicast membership-aware forwarding and snooping behaviour; supports the multicast design and test checklist.

Open source

[S8] RFC 2365: administratively scoped multicast

IPv4 administratively scoped multicast address planning. The example channel allocation is not a deployed Dhruv service.

Open source

[S9] Android Developers: Android Debug Bridge

ADB TCP/IP and wireless debugging. Debugging is conditional engineering access, not a standard signage firewall dependency.

Open source

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av

NILA-AV / DHRUV                       NETWORK SETUP & LIVE STREAMING

24 / NETWORK SETUP

Vendor, hosting and security references

Vendor-hosted endpoints and service behaviour can change. Recheck the appropriate reference at commissioning

and retain the dated, approved site register.

[S10] Let’s Encrypt: challenge types

HTTP-01, DNS-01 and TLS-ALPN-01 requirements; port 80 is conditional, not universally required by a TLS deployment.

Open source

[S11] Hetzner Cloud Firewalls FAQ

Stateful behaviour, inbound/outbound default policy and the documented private Cloud Network limitation. Confirm the actual

Hetzner product and applied resources.

Open source

[S12] Nginx: WebSocket proxying

Upgrade handling and proxy timeout considerations for implemented long-lived routes.

Open source

[S13] Amino: Orchestrate Network Requirements

Vendor page updated 2 April 2026. Source of the conditional Orchestrate/EMEA Resolve destination matrix; grouped firmware entries

require site confirmation.

Open source

[S14] Amino: H200 Quick Start Guide

Vendor page updated 30 June 2026. Provisioning options, basic interfaces and model-dependent power/wireless features.

Open source

[S15] RustDesk Server OSS: Docker

Port roles for ID, NAT test, relay and optional web clients; Pro console distinction.

Open source

[S16] RustDesk Server OSS: installation

Security warning and trusted-proxy restrictions for the optional WebSocket listeners.

Open source

[S17] RFC 4890: filtering ICMPv6

IPv6 firewall considerations, including essential control/error traffic. IPv4 MTU error policy should also be reviewed by the network

owner.

Open source

[S18] IANA protocol numbers

IP protocol-number registry; IGMP uses IP protocol 2, distinct from TCP/UDP port numbers.

Open source

[S19] Haivision: real-time video through a firewall

SRT connection roles, firewall traversal and deployment patterns; media direction is distinct from connection initiation.

Open source

[S20] RFC 3376: IGMP version 3

Membership queries, reports and multicast router state. Use the IGMP version supported by the chosen equipment.

Open source

[S21] RFC 9114: HTTP/3

HTTP over QUIC and handling connectivity failure; HTTP/3 is conditional rather than the base TCP HTTPS profile.

Open source

Dhruv How, What & Why Network Setup Guide | v1.1 | Nila-Av